RELEASE #3 PROCEDURE

The release #3 of the SCA product has been performed using the information coming from MOD bands 02, 04, 06 and from Landsat TM images of the project Corine 2000.

The step of the procedure are shown and explained below.

- Reflectance correction (STEP 1)

The MOD02 bands has been atmospherically corrected using a fixed value of conversion for each band.

- Water masking procedure (STEPs 3, 3a, 4)

- Sum of inland and seawater:

    - -Inland water has been retrieved from CORINE Landsat images.

    - -Seawater from MOD02 threshold

- Water masking from CORINE

- The Corine Landsat images (25m of resolution) has been mosaicked using the projections UTM32 and UTM33 WGS84. the following procedure has been performed twice: one for each projection. Only a cut zone has been considered for every projection.

- The mosaic has been classified with an unsupervised procedure (10 classes and 10 iterations).

- The vector obtained from the unsupervised classification need a procedure of “cleaning” because of the numerous shapes wrongly classified.

- This procedure had to be performed manually.

- Then the vector has been rasterized creating a mask (1;0). Weed tolerance of 100m (every thing under this dimension is ignored)

- The mask has been resampled to the dimension of the MODIS pixel (from 25 to 500 m)

- This procedure of cleaning of object smaller than 5/10/20 MODIS pixels has been performed using the functions CLUMP and ELIMINATE/SIEVE (object smaller or equal to 5 pixels have been eliminated). It is necessary to group pixels adjacent with the CLUMP function (using a filter of 4x4 pixels). The groups over 50 units has been eliminated (function ELIMINATE)

- Therefore, not all the lakes classified has to be used in the water mask because they are not comparable with the pixel size of the MODIS image.

- The mask (raster) has been reprojected to the MODIS images to have a registered mask to be used in the procedure.

- SEA masking from MOD02

Sea boundary has been performed using a threshold for the MOD02 band #2.

- There is a zone of uncertainty around the lakes where pixel could be classified wrongly. So, a buffer has been applied in and out the lake boundaries. The buffer zone depend on the dimension of the object: if the lake is big it is possible to use a big buffer, if it is small, we consider a smaller buffer zone.

- This buffer is of 250m if the lake is small and 500/750m if it is big. For the sea boundary 500 and 750 buffers has been used. These buffered pixel will be use to create a quality flag.

- NDSI and CLOUD masking procedure (STEPs 2, 6, 8, 90)

- NDSI map (with a threshold of 0,5) has been used to select snow and water from the image, but the water has been considered in the previous steps.

- CLOUD masking (with MOD35 data) allows to select cloud coverage and eliminate it in the procedure.

- SCA3 last result

- A final recode has been necessary to have a 5 class image (step 90). Below the legend for the release 3.